# E9XSim CIC Target Interface Hierarchy

Modern CIC-style HMI - working architecture reference

Design basis: Preserve the original CIC first-level domain model and panel-navigation philosophy, but modernize the deeper hierarchy around phone projection, local/USB/Bluetooth media, vehicle integration, telemetry, diagnostics, and the planned E9XSim hardware. The system remains one persistent CIC shell rather than an app launcher.

Key scope decisions: No native navigation engine is planned; Navigation opens phone projection when available. No optical-media UI is included. Local media storage is supported through NVMe and removable USB storage.

# 1. Main Menu Hierarchy

• Main Menu

• Multimedia

• Now Playing

• Artwork / title / artist / album / year / progress

• Playback controls and queue

• Sources

• Bluetooth Audio

• USB Media

• Local Media Library

• Phone Projection Audio

• AUX input (optional, if retained in hardware)

• Bluetooth Audio

• Connected audio device

• Rich metadata / album art

• Play / pause / previous / next

• USB Media

• Browse folders

• Artists / albums / songs

• Playlists

• Search

• Local Media Library

• Artists

• Albums

• Songs

• Playlists

• Recently added

• Search

• Library / storage status

• Radio

• FM

• Station list

• Manual tuning

• Station / program information

• Presets

• AM (optional)

• Station list / manual tuning

• Radio Presets

• Stored stations

• Telephone

• Active Call

• Call status / duration

• Mute microphone

• End call

• Recent Calls

• Incoming

• Outgoing

• Missed

• Dial Number

• Paired Phones

• Connect / disconnect

• Add phone

• Remove / rename / connection roles

• Hands-Free Settings

• Microphone / call-audio status

• Contacts

• All Contacts

• Favorites

• Search

• Contact Details

• Phone numbers

• Call

• Open address in phone projection where supported

• Services

• Phone Projection

• Android Auto

• Launch projection

• Wired connection status

• Wireless connection status

• Apple CarPlay (later / optional)

• Connected Devices

• Phones

• USB devices

• Wireless status

• Software Services

• Current software version

• Software update (later)

• Update / rollback status where supported

• Add-ons (later)

• Installed accessories / integrations

• External gauges / displays

• Ambient-lighting integration

• Approved user modules

• Navigation

• Phone connected with projection available

• Launch the active phone-projection navigation surface directly

• Use the phone projection provider for maps, destination search, routing and traffic

• iDrive controller and touchscreen remain available to projection

• No compatible projection session

• Show phone / projection connection page

• Offer wired or wireless connection methods as configured

• No native map database, route engine or destination-entry hierarchy

• Vehicle Info

• Onboard Info

• Cruising range

• Average fuel consumption

• Average speed

• Trip Computer

• Trip duration

• Trip distance

• Average speed

• Average fuel consumption

• Reset trip values

• Vehicle Status

• Check Control / active warnings

• Stored warning details

• Service Requirements / CBS

• Service items and due dates

• Engine oil level / applicable OEM status

• Tire Monitor

• OEM tire-monitor state

• Reset / initialize

• Vehicle Configuration

• Installed-option awareness / feature availability

• System adapts menus to the coded vehicle configuration

• Performance

• Live Telemetry

• Overview

• Power and Torque

• Live dials / graph

• Estimated / reported values clearly identified

• Boost

• Live boost gauge where applicable to the engine configuration

• Temperatures

• Coolant

• Engine oil

• Transmission where available

• Intake / other useful temperatures where available

• Tire Pressures

• Per-wheel real-time pressure when the vehicle hardware exposes it

• Recorder

• Drive Recorder

• Telemetry Recorder

• Start / stop recording

• Saved sessions

• Session summary / graphs

• Lap Recorder

• Current session

• Lap times / best lap / current lap

• Saved lap sessions

• GNSS-assisted timing when the optional local GNSS module is installed

• Diagnostics

• Overview

• Vehicle module status

• CAN / bus health

• Software / firmware versions

• Faults / Diagnostic Details

• Active faults

• Stored faults

• Fault details / related vehicle state where available

• Live Diagnostic Data

• Selected module / signal details

• System Logs

• Vehicle-interface / communication logs

• CIC boot / crash / service logs

• Service / Developer Mode (development units only)

• SSH / debug access is gated or disabled in production

• Settings

• Vehicle Settings

• Lighting

• Locks / unlocking behavior

• Time / Date

• Clock display format

• Date format

• Automatic synchronization

• Battery-backed RTC provides immediate time before network / phone / GNSS sync

• Vehicle / KOMBI time synchronization

• Units

• Distance / speed / temperature / pressure / consumption

• Climate-related settings where the vehicle exposes them through CIC

• Display

• Brightness

• Automatic day / night behavior

• Split-screen configuration

• Touchscreen settings if needed

• Sound

• Bass / treble

• Balance / fader

• Advanced DSP / EQ controls where supported

• Speed-dependent volume (later)

• Vehicle gong / parking / call level controls as supported

• Connectivity

• Bluetooth

• Wi-Fi

• Android Auto

• Apple CarPlay (later / optional)

• USB devices

• Profiles / Favorites

• Driver profile / stored preferences

• Programmable shortcut / memory-button assignments

• Software / System

• About / version information

• Storage status

• Software update (later)

• Recovery / rollback status where exposed

• Assistant (future / optional)

• Local voice-assistant enable / disable

• Voice / personality profile

• Privacy / local-vs-cloud permissions

# 2. Global / Non-Hierarchical Behavior

• One persistent CIC shell. The UI is one stateful HMI with domains, panel stacks, overlays and services. Opening another domain does not launch or terminate a standalone application.

• MENU is global. MENU can bring the Main Menu over the current context at any time while preserving the previous domain state underneath.

• Direct hard keys switch domains. Radio, media, telephone and navigation shortcuts should jump immediately to their corresponding domains without reinitializing them.

• Navigation is projection-first. The Navigation shortcut enters phone projection and requests its navigation surface when a compatible phone is connected. Without projection, it opens the connection page instead of a native map.

• BACK and controller-left retain CIC-style semantics. BACK follows navigation history; controller-left closes or moves outward through the current panel hierarchy. The exact feel should stay predictable and CIC-like.

• OPTION remains context-sensitive. OPTION opens actions relevant to the current panel, source or selection rather than a generic settings application.

• Domains remember their state. Returning to Multimedia, Radio, Vehicle Info, Performance or another domain should generally restore the last useful panel and selection.

• Split screen is persistent secondary content. A configurable secondary region can remain visible while the primary domain changes. Native vehicle/media data can use it freely; coexistence with phone projection depends on the projection surface and integration limits.

• The status area is persistent. Time, active media/source state, phone/projection connection, warnings and other concise system state can remain visible across native CIC panels.

• Media playback is independent of the visible menu. Changing panels or entering Vehicle Info / Settings does not stop the selected radio, Bluetooth, USB, local or projected audio source.

• Projection is a surface inside the CIC experience, not the operating-system shell. Phone projection can own the main content region when active, while the CIC still owns vehicle integration, source routing, hardware controls and safety-critical overlays.

• Reverse camera / PDC can override normal hierarchy. Selecting reverse immediately presents the low-latency camera view and PDC visualization. This path should be available before the full normal UI has finished starting.

• Check Control and call events can interrupt normally. Warnings, incoming calls and other high-priority events appear as overlays or temporary foreground panels and return to the previous context when resolved.

• Touch and iDrive are peers. Every normal native CIC function should remain usable from the physical controller; touch is an additional direct-control path rather than a replacement.

• Steering-wheel controls remain global. Volume, track/station, phone and voice controls operate regardless of the visible native panel when their action is applicable.

• The clock is hardware-backed. A battery-backed RTC supplies correct time immediately at boot. Network, phone or optional GNSS time can later discipline the clock, and CIC keeps the vehicle/KOMBI time synchronized.

• Fast wake is part of the UX. The device should wake into a useful display quickly, resume cleanly after short stops and enter a low-power parked state without exposing Linux boot or desktop UI.

• Future voice assistance is shell-level. A local assistant, if added, is invoked globally and can query or call approved CIC/vehicle capabilities. It is not a separate full-screen app and is not allowed direct safety-critical vehicle control.

• No general-purpose app-launcher dependency. New E9XSim features should normally appear as new panels, services or approved add-ons inside the CIC hierarchy rather than turning the product into a tablet-style home screen.

# 3. Structural Notes / Boundaries

• Original top-level model retained. The original CIC domains are kept in recognizable form: Multimedia, Radio, Telephone, Contacts, Services, Navigation, Vehicle Info and Settings. Performance and Diagnostics are added as first-class E9XSim domains.

• Multimedia replaces the legacy optical-media emphasis. USB, Bluetooth, phone projection and local NVMe storage are the intended media sources.

• Navigation intentionally has very little native hierarchy. Google Maps, Waze or another projection navigation provider owns maps, search, route calculation and traffic. E9XSim only handles connection, launch, controls and audio integration.

• Performance is native and vehicle-aware. Telemetry, recorders, tire pressures, temperatures, boost and power/torque views are native CIC functions because they depend on E9XSim vehicle data rather than phone applications.

• Diagnostics is native but separated from normal vehicle status. Vehicle Info remains the owner-facing status area; Diagnostics is the deeper technical/service area for faults, module state and communication details.

• Drive Recorder is treated as a vehicle/telemetry session feature. Optional camera-based dashcam recording remains a separate future camera capability rather than being assumed by the V1 Drive Recorder entry.

• Add-ons remain controlled integrations. Future external gauges, displays, ambient lighting and approved modules integrate through E9XSim capabilities instead of creating an unrestricted third-party application ecosystem.

# 4. Source Basis

Primary project source: E9XSim_CIC_Feature_Requirements.xlsx, especially the edited “E9XSim CIC Target” sheet and its V1 enthusiast additions.